Tradeoff Analysis

The following tradeoff analysis documents the various technologies considered for various system functions, broken down by component: the Personnel Locator (PL), the Signal Posts (SP), and the Command Center (CC).

Personnel Locator

Location Determination

- Cellular Base Station Triangulation

- Global Positioning System

- PATS-specific Radio Frequency based Tri-lateration with Signal Posts

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - Terrestrial-based - Infrastructure available in populated areas - Standards are published and open - Line of sight not required - Hardware is readily available | - No global standard, only regional - Remote areas do not have coverage - Need collaboration with base station owners - May incur monetary costs |
| 2 | - Available worldwide - Maintained by the U.S. Air Force - Passive device, no collaboration with equipment owners needed - Free to receive GPS signals - Hardware is readily available | - Line of sight required |
| 3 | - Mission-specific design - RF design well understood by many contractors (i.e. Motorola, Texas Instruments, Kenwood, etc.) - Line of sight not required | - New implementation; design not ubiquitous |

Communications

- Infrared Optical

- Radio Frequency Single Frequency Band (Digital)

- Radio Frequency Single Frequency Band (Analog)

- Radio Frequency Multiple Frequency Band (Digital)

- Radio Frequency Multiple Frequency Band (Analog)

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - IrDA specifications/standards available - Hardware is readily available | - Line of sight required - Susceptible to disturbances - Distance is limited |
| 2 | - Mission-specific design - Digital communications preserves battery life - RF design well understood by many contractors | - New implementation; design not ubiquitous - Single RF frequency band limits flexibility and performance |
| 3 | - Mission-specific design - Analog communications requires more power - RF design well understood by many contractors | - New implementation; design not ubiquitous - Single RF frequency band limits flexibility and performance |
| 4 | - Mission-specific design - Digital communications preserves battery life - Multi-band RF provides flexibility - RF design well understood by many contractors | - New implementation; design not ubiquitous |
| 5 | - Mission-specific design - Analog communications requires more power - Multi-band RF provides flexibility - RF design well understood by many contractors | - New implementation; design not ubiquitous |

  - Ultra Low Frequency

  - Very High Frequency

Table 1:  RF Technologies Trade

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - Can penetrate water and earth - Currently used in TTE communications for mine safety | - Slow data rate - Susceptible to electronic interference |
| 2 | - Currently used in broadcast television - Higher robustness to atmospheric and earth interference - Higher data rates | - Less able to penetrate earth and water |

Identification

- Internal Programmable Flash Memory

- Internal Read-Only Memory

- Removable Flash Memory

- Radio Frequency Identification (RFID)

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - Simple design - Can be programmed - Internal to PL | - Replacement requires disassembly - Programming interface needs to be designed - Cannot detect if “PL separated from wearer” |
| 2 | - Simple design - Internal to PL | - Replacement requires disassembly - Once programmed, contents not easily changed - Cannot detect if “PL separated from wearer” |
| 3 | - Simple design - Many removable flash memory products exist - Can be moved easily from one PL to another - Can be written via card reader | - Removable parts are easily lost - Additional item to be carried - Cannot detect if “PL separated from wearer” |
| 4 | - Simple design - Incorporated into employee ID badge - RFID a well established technology - Allows for detection of “PL separated from wearer” | - Programming requires special equipment |

Distress Notification

- Autonomous

- Manual

- Hybrid (Autonomous and Manual)

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - No user interaction is necessary - No input interface needed | - Prone to false alarms |
| 2 | - Less prone to false alarms | - User interaction is necessary - Input interface required |
| 3 | - Allows user to manually issue a distress notification - Allows user to manually disable a distress notification | - User interaction may be required |

Display

- No Display Provided

- Light Emitting Diodes

- Liquid Crystal Display

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - Simple enclosure design - Easily ruggedized | - No user insight into PL operational status |
| 2 | - Simple enclosure design | - Limited user insight into PL operational status |
| 3 | - Provides maximum insight into PL operational status - Allows user to manually disable a distress notification | - Complex enclosure design - Difficult to ruggedize |

Signal Posts

Location Determination

- Cellular Base Station Triangulation

- Global Positioning System

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - Terrestrial-based - Infrastructure available in populated areas - Standards are published and open - Line of sight not required - Hardware is readily available | - No global standard, only regional - Remote areas do not have coverage - Need collaboration with base station owners - May incur monetary costs |
| 2 | - Available worldwide - Maintained by the U.S. Air Force - Passive device, no collaboration with equipment owners needed - Free to receive GPS signals - Hardware is readily available | - Line of sight required |

Communications

- Infrared Optical

- Radio Frequency Single Frequency Band (Digital)

- Radio Frequency Single Frequency Band (Analog)

- Radio Frequency Multiple Frequency Band (Digital)

- Radio Frequency Multiple Frequency Band (Analog)

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - IrDA specifications/standards available - Hardware is readily available | - Line of sight required - Susceptible to disturbances - Distance is limited |
| 2 | - Mission-specific design - Digital communications preserves battery life - RF design well understood by many contractors | - New implementation; design not ubiquitous - Single RF frequency band limits flexibility and performance |
| 3 | - Mission-specific design - Analog communications requires more power - RF design well understood by many contractors | - New implementation; design not ubiquitous - Single RF frequency band limits flexibility and performance |
| 4 | - Mission-specific design - Digital communications preserves battery life - Multi-band RF provides flexibility - RF design well understood by many contractors | - New implementation; design not ubiquitous |
| 5 | - Mission-specific design - Analog communications requires more power - Multi-band RF provides flexibility - RF design well understood by many contractors | - New implementation; design not ubiquitous |

Table 2:  RF Technologies Trade1

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - Can penetrate water and earth - Currently used in TTE communications for mine safety | - Slow data rate - Susceptible to electronic interference |
| 2 | - Currently used in broadcast television - Higher robustness to atmospheric and earth interference - Higher data rates | - Less able to penetrate earth and water |

Command Center

Communications

- Infrared Optical

- Radio Frequency Single Frequency Band (Digital)

- Radio Frequency Single Frequency Band (Analog)

- Radio Frequency Multiple Frequency Band (Digital)

- Radio Frequency Multiple Frequency Band (Analog)

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - IrDA specifications/standards available - Hardware is readily available | - Line of sight required - Susceptible to disturbances - Distance is limited |
| 2 | - Mission-specific design - Digital communications preserves battery life - RF design well understood by many contractors | - New implementation; design not ubiquitous - Single RF frequency band limits flexibility and performance |
| 3 | - Mission-specific design - Analog communications requires more power - RF design well understood by many contractors | - New implementation; design not ubiquitous - Single RF frequency band limits flexibility and performance |
| 4 | - Mission-specific design - Digital communications preserves battery life - Multi-band RF provides flexibility - RF design well understood by many contractors | - New implementation; design not ubiquitous |
| 5 | - Mission-specific design - Analog communications requires more power - Multi-band RF provides flexibility - RF design well understood by many contractors | - New implementation; design not ubiquitous |

Computer Hardware

- PATS-specific Proprietary Designed Computer

- Standard x86-based Computer

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - Mission-specific design | - Special training maybe required - Uncertain upgrade path - Design may me lengthy - May require a new operating system - Expensive |
| 2 | - Standardized equipment - Readily available operating system choices - New hardware continually released - Cost effective: no new design needed | - Hardware selected during the requirements phase maybe obsolete by the implementation phase |

Operating System

- Linux

- Macintosh OS

- Microsoft Windows

| Option | Strengths | Weaknesses |
| --- | --- | --- |
| 1 | - Open source - Large developer base - Runs on standard x86 hardware architecture | - Can lack polish - Community based support structure |
| 2 | - Polished and consistent user interface | - Closed source - Limited developer base - Requires Apple hardware |
| 3 | - Polished and consistent user interface - Large developer base - User support backed by a large corporation - Runs on standard x86 hardware architecture | - Closed source |

"Radio frequency." Wikipedia, The Free Encyclopedia. 19 Oct 2009, 08:17 UTC. 19 Oct 2009 <http://en.wikipedia.org/w/index.php?title=Radio_frequency&oldid=320760634>.